PROJECT T + ZENO

Detailed Technical Status, Architecture & Development Roadmap

Prepared: 28 September 2026
Self-Balancing Intelligent Companion Robot

# 1. Executive Summary

Project T is an intelligent mobile companion robot. Zeno is its AI voice assistant and high-level interaction layer. The long-term system combines conversational AI, speech recognition/synthesis, computer vision, person recognition, autonomous movement, obstacle avoidance and a self-balancing robotic platform.

The project has progressed beyond the concept stage. The mechanical prototype, Raspberry Pi software environment, audio input/output chain, Gemini text interaction, wake-word testing, camera interface, ESP32, MPU6050 and motor-control hardware are available or working. The main current software bottleneck is stable multi-turn Gemini Live session and audio-stream lifecycle management.

For the seminar, the project should be presented as an actively developed prototype. Demonstrate verified subsystems and clearly label balancing, Follow-Me and autonomous navigation as development stages.

# 2. Project Vision

Wake-word activation using “Hey Zeno”.

Natural multi-turn voice conversation without repeating the wake word.

Low-latency conversation with future interruption/barge-in support.

General question answering through AI.

Camera-based person and face recognition using Raspberry Pi Camera Module 3 Wide.

Consent-based onboarding and memory of permitted information.

Personalized greetings for known people.

Follow-Me: identify a target, track them, maintain distance and command motors.

Future OCR, object recognition and scene understanding.

Self-balancing movement using IMU, encoders and closed-loop control.

Future obstacle avoidance and autonomous navigation.

# 3. System Architecture

The Raspberry Pi is the high-level brain; the ESP32 is the real-time controller. This separation prevents AI/network workloads from directly controlling safety-critical motor timing.

| Subsystem | Controller | Responsibilities |
| --- | --- | --- |
| AI / Voice | Raspberry Pi | Wake word, STT, Gemini, TTS, audio, conversation |
| Vision | Raspberry Pi | Camera, recognition, OCR, tracking |
| High-level Brain | Raspberry Pi | Decisions, memory, robot commands |
| Real-time Control | ESP32 | IMU, encoders, PID, PWM, motor safety |
| Drive | ESP32 + RMCS-2305 | Direction, speed and enable control |
| Safety | ESP32 | Fast stop and fault handling |

# 4. Raspberry Pi Platform

Raspberry Pi 3 Model B, 1 GB RAM.

64-bit Raspberry Pi OS / Debian 13 environment.

Kernel: 6.18.50+rpt-rpi-v8.

Python 3.13.5.

Project root: ~/ProjectT/.

VS Code Remote SSH is the development workflow.

GitHub repository has been initialized and pushed.

Power integrity remains a known issue. The Pi has shown undervoltage/throttling indications and approximately 4.7 V at the 5 V pins in some tests. A stable 5 V supply with sufficient current capacity and motor-noise isolation remains important before the final mobile demonstration.

# 5. Audio System

## 5.1 INMP441 Microphone

VDD → Pi 3.3 V; GND → common ground.

SCK → GPIO18 / physical pin 12.

WS → GPIO19 / physical pin 35.

SD → GPIO20 / physical pin 38.

L/R → GND.

Capture has been verified; the Live path uses 16 kHz, 16-bit, mono PCM.

## 5.2 MAX98357A Speaker

LRC → GPIO19 / pin 35.

BCLK → GPIO18 / pin 12.

DIN → GPIO21 / pin 40.

VIN → 5 V; GND → common ground.

Speaker connects between OUT+ and OUT−, never to Pi GND.

Direct aplay playback works.

Gemini-generated 24 kHz mono audio has been successfully played.

Therefore, the microphone, speaker and basic ALSA playback path are not the primary cause of the Gemini Live multi-turn failure.

# 6. Zeno Voice AI

## 6.1 Working Components

faster-whisper speech-to-text.

Edge-TTS speech synthesis.

Gemini normal text API.

Zeno wake-word test.

Gemini Live single-turn audio interaction.

Buffered Gemini audio can be saved and played successfully.

The proven sequential fallback is: microphone → Whisper → Gemini text → Edge-TTS → MAX98357A.

## 6.2 Gemini Live Multi-Turn Problem

The current bottleneck is continuous multi-turn Live operation. The first turn can work, but the next turn may fail with a WebSocket/session error:

ConnectionClosedError: sent 1011 (internal error) keepalive ping timeout; no close frame received

Testing indicates that basic connectivity, microphone capture and speaker playback work independently. The evidence therefore points toward Live session/audio-stream lifecycle handling as the main area to fix.

## 6.3 Immediate Live Test

A targeted next experiment is to explicitly signal the temporary end of the microphone stream at turn completion:

await session.send_realtime_input(audio_stream_end=True)

If this does not solve the issue, simplify the session lifecycle, separate microphone streaming from turn state, implement controlled reconnect/session resumption, and retain the sequential voice pipeline as a fallback.

# 7. Camera and Vision

Raspberry Pi Camera Module 3 Wide is installed and tested.

The IMX708 wide sensor is detected successfully.

Still-image capture has been verified.

Planned uses: face/person recognition, OCR, tracking and scene understanding.

A fixed wide camera cannot provide true simultaneous 360° vision. Future full-area coverage would require pan/tilt, multiple cameras or a panoramic solution.

# 8. ESP32 Real-Time Control

The ESP32-WROOM-32 is intended for low-level control because balancing and motor control require predictable timing.

ESP32-WROOM-32 with CP2102.

MPU6050 through I2C.

RMCS-2305 dual motor driver.

Two 12 V, 100 RPM geared DC motors with quadrature encoders.

Encoder inputs planned on GPIO32–35.

Motor control signals planned on GPIO12–27.

# 9. MPU6050 Integration

SDA → GPIO18; SCL → GPIO19.

VCC → 3.3 V; GND → common ground.

I2C address detected: 0x68.

Pitch estimation using a complementary filter works.

Current calibration offsets are installation-specific and should be recalibrated if the sensor mounting changes.

# 10. Motor Driver and Encoders

## 10.1 RMCS-2305

6–30 V operating range.

Dual motor driver, approximately 20 A continuous/channel.

Battery +12 V → U+; battery negative → U−.

ESP32 and driver share common logic ground.

Driver +5 V is an output, not a 5 V input.

SLEEP pins are used for enable/disable.

## 10.2 Encoders

Each motor has quadrature A/B feedback.

Encoder supply is 5 V.

ESP32 GPIO is 3.3 V, so level reduction is required.

Planned divider: 5.1 kΩ series + 10 kΩ to ground at the ESP32 input.

Encoder integration is the next major low-level milestone.

# 11. Self-Balancing Roadmap

Complete reliable quadrature encoder acquisition.

Verify wheel direction and counts.

Implement motor speed control.

Combine IMU pitch/rate with encoder feedback.

Add safety stop and restrained testing.

Tune PID gradually.

Validate disturbance recovery.

Only then accept high-level motion commands from the Pi.

Balancing should remain on the ESP32. The Pi should send high-level desired motion or mode commands.

# 12. Follow-Me Feature

Follow-Me is a multi-stage system:

Activate Follow-Me through Zeno.

Detect/identify the target person.

Track the target across frames.

Estimate relative position/distance using available sensing.

Generate safe movement commands.

Send high-level commands to the ESP32.

ESP32 performs balancing and wheel control.

Stop if target is lost, an obstacle is detected, a safety condition occurs, or the user says stop.

The first implementation should prioritize reliable tracking and safe stopping. Obstacle avoidance and full navigation can be added later.

# 13. Memory and Person Recognition

Unknown-person onboarding should be consent based.

Ask for a name and only store information the person permits.

Known-person recognition can trigger personalized greetings.

Keep memory separate from the AI provider so the backend can be changed later.

# 14. Recommended Software Architecture

| Layer | Purpose |
| --- | --- |
| Voice Interface | Mic, wake word, STT, playback |
| AI Provider | Gemini Live/normal API and future providers |
| Conversation Manager | Turn-taking, context, interruption, session state |
| Memory | Consent-based persistent information |
| Vision | Camera, face recognition, OCR, tracking |
| Robot Command Bus | Standard movement/mode commands |
| ESP32 Controller | Balancing, PID, encoders, PWM, safety |

This provider-independent architecture prevents Gemini-specific logic from spreading through the whole robot.

# 15. GitHub and Development Practice

ProjectT repository is initialized and pushed to GitHub.

Main branch tracks the remote.

Sensitive .env credentials are excluded by .gitignore.

Audio/image/compiled artifacts are excluded where appropriate.

Use small descriptive commits to preserve known-good states.

Collaborators should use their own GitHub accounts rather than sharing the owner's password.

# 16. Power System

Main battery: 12 V, 20 Ah lithium-ion pack.

Motor driver powered from the battery.

Separate regulated 5 V path intended for Pi/logic.

Resolve undervoltage before final demonstration.

Keep high-current motor paths isolated from sensitive computer/audio power as far as practical while maintaining a proper signal ground.

# 17. Current Status Snapshot

| Area | Status | Notes |
| --- | --- | --- |
| Mechanical chassis | Working / substantially complete | Cutting, drilling, motor and battery integration completed |
| Raspberry Pi | Working | Power stability still needs attention |
| Microphone | Working | INMP441 verified |
| Speaker | Working | MAX98357A verified |
| Whisper / Edge-TTS | Working | Sequential voice path available |
| Gemini normal API | Working | Text interaction verified |
| Gemini Live single turn | Working | Audio response verified |
| Gemini Live multi-turn | In progress | Lifecycle/session instability |
| Wake word | Working in test | Zeno detection demonstrated |
| Camera | Working | Wide camera tested |
| ESP32 | Integrated/available | Low-level controller |
| MPU6050 | Working | I2C + pitch estimation |
| Motor driver | Available/testing | RMCS-2305 |
| Encoders | Next | Level-shifted A/B inputs planned |
| PID balancing | Next | After encoder integration |
| Follow-Me | Future | Vision + control integration |
| Obstacle avoidance | Future | Sensor/vision integration |
| Autonomous navigation | Future | After stable balancing/movement |

# 18. Immediate Development Plan

Test audio_stream_end at Live turn completion.

If necessary, simplify Live before adding features.

Keep sequential voice as fallback.

Complete encoder wiring and quadrature verification.

Implement motor speed control.

Integrate pitch estimation with controlled balancing tests.

Create Pi↔ESP32 command protocol.

Create commands such as STOP, MOVE, FOLLOW and BALANCE.

Add person detection/tracking.

Implement Follow-Me only after safe movement and emergency stop are reliable.

Add obstacle detection and autonomous navigation afterward.

Resolve Pi power stability before final demonstration.

# 19. Seminar Demonstration Strategy

Show the physical prototype and architecture.

Demonstrate Zeno voice interaction using the reliable path.

Show the Camera Module 3 Wide.

Show ESP32, RMCS-2305 and motor hardware.

Demonstrate MPU6050 detection/pitch estimation if practical.

Explain encoder integration and PID balancing as the next control milestone.

Present Follow-Me, obstacle avoidance and navigation as future development.

This presents a credible progression: mechanical prototype → sensing → AI voice → real-time control → balancing → perception → autonomous behavior.

# 20. Proposed 12-Slide Presentation

Title — Project T: Development of a Self-Balancing Intelligent Companion Robot

Motivation — Need for an intelligent physical companion capable of interaction and movement

Objectives — Voice AI, vision, balancing, person following and autonomy

System Architecture — Raspberry Pi high-level brain + ESP32 real-time controller

Mechanical Design — Chassis, wheels, geared motors and battery

Electronics — Pi, ESP32, IMU, driver, audio and camera

AI Voice System — Zeno, microphone, Gemini, STT/TTS and speaker

Vision System — Camera, recognition and tracking roadmap

Motion Control — Encoders, motor driver, IMU and PID

Current Development — Working subsystems and current Live/encoder work

Future Work — Balancing, Follow-Me, obstacle avoidance, navigation

Conclusion — Integrated prototype and path toward autonomous companion

# 21. Key Engineering Principles

Keep safety-critical control on the ESP32.

Never allow AI-generated commands to directly bypass a safety/control layer.

Use explicit states such as IDLE, LISTENING, SPEAKING, FOLLOWING, BALANCING and STOP.

Maintain a fast STOP path.

Treat feedback prevention as part of voice architecture.

Keep the AI provider behind an interface.

Integrate one subsystem at a time.

Use Git to preserve known-good states.

Do not claim a feature is complete until physically demonstrated.

# 22. Overall Assessment

Project T has reached a genuine integrated prototype stage. The major building blocks are present: physical platform, embedded controller, IMU, motor driver, encoders, Raspberry Pi, digital audio, AI interaction and camera hardware.

The immediate software priority is stable continuous voice interaction. The immediate robotics priority is encoder integration followed by closed-loop motor control and balancing. Once these foundations are stable, person tracking and Follow-Me can be added without mixing high-level AI decisions with low-level motor safety.

Recommended sequence: reliable voice/session layer → encoder feedback → motor control → balancing → Pi/ESP32 command bus → person tracking → Follow-Me → obstacle avoidance → autonomous navigation.

# Appendix — Key Technical Paths

Project root:
~/ProjectT/

Virtual environment:
source ~/ProjectT/.venv/bin/activate

Microphone:
plughw:1,0

Speaker:
plughw:1,1

Live input:
16-bit PCM, 16 kHz, mono

Live output:
16-bit PCM, 24 kHz, mono

Audio chunk:
1280 bytes ≈ 40 ms

Current Live model under test:
gemini-3.1-flash-live-preview

GitHub:
https://github.com/TrilochanParida/ProjectT

END OF DOCUMENT